For combinations cells (depending on number of inputs – example, inverter has only A input), the following characterizations have to be performed and filled. Remove all unwanted rows.

- Input pin capacitances:

| Input Pins | Rise Cap (pF) | Fall Cap (pF) | Average Cap (pF) |
| --- | --- | --- | --- |
| A | 0.0024 | 0.0023 | 0.0023 |

- Transition Time Table: (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.0169 | 0.0171 | 0.0324 |
| 10 fF | 0.0722 | 0.0722 | 0.0776 |
| 100 fF | 0.6136 | 0.6136 | 0.6143 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.0172 | 0.0175 | 0.0305 |
| 10 fF | 0.0757 | 0.0757 | 0.0819 |
| 100 fF | 0.6444 | 0.6444 | 0.6455 |

- Propagation delay time tables: (unlike textbook definitions that we used for our assignments, here we will use 50% of input to 50% of output to simulate propagation delay – by keeping other inputs fixed).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.0478 | 0.0661 | 0.1430 |
| 10 fF | 0.0886 | 0.1069 | 0.1921 |
| 100 fF | 0.4577 | 0.4762 | 0.5624 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.0482 | 0.0693 | 0.1568 |
| 10 fF | 0.0970 | 0.1184 | 0.2135 |
| 100 fF | 0.5307 | 0.5531 | 0.6516 |

- Static Power (cover all possible input combinations based on number of inputs).

| Condition (A) | Power (nW) |
| --- | --- |
| 0 | 0.135 |
| 1 | 0.135 |

- Dynamic Power Table:

(i) Rise Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 739.3 | 783.2 | 793.1 |
| 10 fF | 649.2 | 691.7 | 711 |
| 100 fF | 765.5 | 780.5 | 816.6 |

(ii) Fall Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 1409.8 | 1461.4 | 1472.8 |
| 10 fF | 1335.7 | 1394.7 | 1421.1 |
| 100 fF | 1498.3 | 1536.6 | 1597.9 |