For combinations cells (depending on number of inputs – example, inverter has only A input), the following characterizations have to be performed and filled. Remove all unwanted rows.

- Input pin capacitances:

| Input Pins | Rise Cap (pF) | Fall Cap (pF) | Average Cap (pF) |
| --- | --- | --- | --- |
| A | 0.00225697 | 0.002169711 | 0.002213341 |
| B | 0.002921491 | 0.002865751 | 0.002893621 |

- Transition Time Table: (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.02198 | 0.02208 | 0.0373 |
| 10 fF | 0.07724 | 0.07725 | 0.08225 |
| 100 fF | 0.61861 | 0.61861 | 0.61912 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.02958 | 0.03219 | 0.1132 |
| 10 fF | 0.08625 | 0.08621 | 0.19255 |
| 100 fF | 0.62807 | 0.62807 | 0.63745 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.01975 | 0.01995 | 0.03353 |
| 10 fF | 0.06826 | 0.06829 | 0.07577 |
| 100 fF | 0.53635 | 0.53635 | 0.53738 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.01824 | 0.02060 | 0.079 |
| 10 fF | 0.06793 | 0.06802 | 0.137 |
| 100 fF | 0.53635 | 0.53635 | 0.54708 |

- Propagation delay time tables: (unlike textbook definitions that we used for our assignments, here we will use 50% of input to 50% of output to simulate propagation delay – by keeping other inputs fixed).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 56.95 | 75.55 | 165.10 |
| 10 fF | 98.67 | 117.11 | 211.97 |
| 100 fF | 468.50 | 487.27 | 582.86 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 47.81 | 66.14 | 143.03 |
| 10 fF | 88.69 | 106.97 | 192.15 |
| 100 fF | 457.76 | 476.24 | 562.42 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 51.23 | 72.68 | 166.20 |
| 10 fF | 91.16 | 112.81 | 213.66 |
| 100 fF | 443.88 | 466.44 | 570.51 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 48.22 | 69.37 | 156.80 |
| 10 fF | 97.07 | 118.47 | 213.57 |
| 100 fF | 530.70 | 553.18 | 651.61 |

- Static Power (cover all possible input combinations based on number of inputs).

| Condition (AB) | Power (nW) |
| --- | --- |
| 00 | 0.019 |
| 01 | 0.138 |
| 10 | 0.267 |
| 11 | 0.138 |

- Dynamic Power Table:

(i) Rise Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 1280.82 | 1176.94 | 1296.64 |
| 10 fF | 5232.77 | 5121.71 | 5175.5 |
| 100 fF | 41731.2 | 41639.9 | 41731.9 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 2275.3 | 2190.6 | 2379.82 |
| 10 fF | 2335.4 | 2260.82 | 2424.77 |
| 100 fF | 2354.46 | 2297.92 | 2506.36 |

(ii) Fall Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 1843.66 | 1756.34 | 2025.01 |
| 10 fF | 5698.57 | 5636.75 | 5785.47 |
| 100 fF | 42147.7 | 42121.3 | 42074.3 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 621.407 | 531.801 | 577.916 |
| 10 fF | 640.402 | 594.285 | 546.071 |
| 100 fF | 646.356 | 638.745 | 598.791 |