For Sequential cells, the following characterizations have to be performed and filled.

- Input pin capacitances:

| Input Pins | Rise Cap (pF) | Fall Cap (pF) | Average Cap (pF) |
| --- | --- | --- | --- |
| D | 0.00222474 | 0.00216311 | 0.002193925 |
| CLK | 0.00841449 | 0.00839373 | 0.00840411 |

- Set-up Time Table:

(i) Rise Constraint (in ns) [Input slew vs CLK slew].

| D / CLK | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0.121 | 0 |
| 1000 ps | 0.782 | 0.165 |

(ii) Fall Constraint (in ns) [Input slew vs CLK slew].

| D / CLK | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0.130 | 0 |
| 1000 ps | 0.802 | 0.178 |

- Hold Time Table:

(i) Rise Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | -0.1209 | -0.01019 |
| 1000 ps | -0.2565 | -0.1645 |

(ii) Fall Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | -0.1295 | -0.2105 |
| 1000 ps | -0.3065 | -0.3755 |

- Transition Time Table (for Q output, CLK will have minimum slew of 10 ps): (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.0203 | 0.0204 | 0.0205 |
| 10 fF | 0.0709 | 0.0712 | 0.0713 |
| 100 fF | 0.5975 | 0.5977 | 0.5979 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.0179 | 0.01799 | 0.01825 |
| 10 fF | 0.07530 | 0.07534 | 0.07538 |
| 100 fF | 0.64083 | 0.64087 | 0.64092 |

- CLK-to-Q Delay Time Table: (delay between clock transition and data transition. Use 50% of CLK to 50% of output to simulate propagation delay).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.19280 | 0.19300 | 0.1932 |
| 10 fF | 0.24210 | 0.24340 | 0.24360 |
| 100 fF | 0.60170 | 0.6067 | 0.60722 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.13778 | 0.13794 | 0.1382 |
| 10 fF | 0.19653 | 0.19654 | 0.19717 |
| 100 fF | 0.63343 | 0.63349 | 0.63356 |

- Static Power (all possible input combinations of CLK and D).

| Condition (CLK, D) | Power (nW) |
| --- | --- |
| 00 | 67778.58 |
| 01 | 0.3676506 |
| 10 | 0.4729598 |
| 11 | 0.4729599 |

- Dynamic Power Table: (CLK will have minimum slew of 10 ps)

(i) Rise Power (in nW) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 15285 | 14714 | 14767 |
| 10 fF | 18954 | 18385 | 18447 |
| 100 fF | 47578 | 47009 | 47073 |

(ii) Fall Power (in nW) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 11077 | 11077 | 11077 |
| 10 fF | 10765 | 10764 | 10764 |
| 100 fF | 11290 | 11290 | 11290 |